Meta-Surface Wall Suppression of Mutual Coupling Between Microstrip Patch Antenna Arrays for THz-Band Applications

Mohammad Alibakhshikenari1*, Bal S. Virdee2, P. Shukla2, Chan H. See3, Raed Abd-Alhameed4, Francisco Falcone5, and Ernesto Limiti1

1 Electronics Engineering Department, University of Rome “Tor Vergata”, Via del politecnico 1, 00133, Rome, ITALY

2 London Metropolitan University, Center for Communications Technology & Mathematics, School of Computing & Digital Media, London N7 8DB, UK

3 School of Engineering, University of Bolton, Deane Road, Bolton, BL3 5AB, UK

4 School of Electrical Engineering & Computer Science, University of Bradford, UK

5 Electric and Electronic Engineering Department, Universidad Pública de Navarra, SPAIN

* alibakhshikenari@ing.uniroma2.it

Abstract- This paper presents a novel 2D meta-surface wall to increase the isolation between microstrip patch radiators in an antenna array that is operating in the teraherz (THz) band of 139-141 GHz for applications including communications, medical and security screening systems. The meta-surface unit-cell comprises conjoined twin ‘Y-shaped’ microstrip structure which are inter-digitally interleaved together to create the meta-surface wall. The proposed meta-surface wall is free of via holes and defected ground-plane hence easing its fabrication. The meta-surface wall is inserted tightly between the radiating elements to reduce surface wave mutual coupling. For best isolation performance the wall is oriented orthogonal to the patch antennas. The antenna array exhibits a gain of 9.0 dBi with high isolation level of less than -63 dB between transmit and receive antennas in the specified THz-band. The proposed technique achieves mutual coupling suppression of more than 10 dB over a much wider frequency bandwidth (2 GHz) than achieved to date. With the proposed technique the edge-to-edge gap between the transmit and receive patch antennas can be reduced to 2.5 mm. Dimensions of the transmit and receive patch antennas are 5×5 mm2 with ground-plane size of 9×4.25 mm2 when constructed on a conventional lossy substrate with thickness of 1.6 mm.

Keywords- Meta-surface wall isolator, antennas, THz-band, mutual coupling suppression, densely packed arrays.

- INTRODUCTION

Microstrip patch antennas have become popular for applications in wireless communication systems as they possess desirable attributes of low cost, lightweight, and planar configuration, and ease of integration. In antenna arrays the radiating elements need to be in close proximity to each other to realise a small form factor, which is highly desirable in modern wireless systems. This requirement however causes significant mutual coupling effects that inevitably degrades the performance of the antenna array. Isolation between neighbouring radiating elements in the antenna can be improved by simply increasing the gap between the elements, but this is at the cost of increased antenna size [1]-[3]. To overcome the size issue, the patch antenna can be printed on high dielectric substrates, but the resulting surface waves can significantly deteriorate the radiation characteristics of the antenna. This is because on a finite ground-plane the surface waves are reflected and diffracted at the edges of the substrate, which results in a significant amount of energy loss [4][5]. To date numerous techniques have been explored to improve the radiation performance of patch antennas implemented on substrates [6]-[8]. This includes incorporating photonic bandgap (PBG) or electromagnetic bandgap (EBG) structures around the radiating elements [9]-[12]. Another approach to suppress surface waves is to use artificial soft and hard surfaces realized with EBG [13]. The soft surface behaves like a perfect electric conductor (PEC) in H-plane and as a perfect magnetic conductor (PMC) in E-plane, and visa-versa for the hard surface. Soft surfaces exhibit bandgaps in only one direction, and are created on the ground-plane of a microstrip patch antenna. These results show that the presence of a via close to a radiating element can affect its resonant frequency. In addition, a large surface area is required to implement EBG structures.

This paper presents a unique method for enhancing the isolation between microstrip patch radiators for application in antenna arrays, where the transmit/receive bands are very close to each other. Reduction in mutual coupling between adjacent radiating elements that are on the same plane is achieved with a novel 2D meta-surface wall isolator (MSWI) analogous to [14]. In the proposed technique the meta-surface does not require any ground-plane or metallic vias. The proposed MSWI is located between radiating elements. It consists of conjoined twin ‘Y-shaped’ structures that are inter-digitally interleaved with each other to create a meta-surface wall. A 1×2 microstrip patch antenna array design with the proposed meta-surface wall is investigated in the THz-band from 139 GHz to 141 GHz to demonstrate the feasibility of the proposed technique.

- META-SURFACE WALL ISOLATOR STRUCTURE

Geometry of the 1×2 microstrip patch antenna array using the proposed meta-surface wall isolator is shown in Fig. 1. The meta-surface consists of inter-digitally coupled unit-cell structure of conjoined twin Y-shaped configuration. The meta-surface wall, shown in Fig. 1(b), comprises several unit-cell structures that are stacked together vertically at regular distances to create the appearance of a different bulk propagation medium. Each conjoined twin Y-shaped structure acts like a resonant circuit. As long as the inserted unit-cell structures are small compared to the propagating wavelength, they create a macroscopic effect (permittivity and permeability) on the electromagnetic wave as it passes through it. The meta-surface is designed to highly impede electromagnetic propagation along the antenna array’s surface within the specified THz frequency band of operation. It is effectively an artificial high-impedance surface that blocks current from flowing in the specified THz frequency band. Hence, the proposed structure is used here as an effective isolator that suppresses mutual coupling between the two radiators, as shown in Fig. 1(c). MSWI is oriented orthogonal to the radiators for effective suppression of mutual coupling between the Tx/Rx. The physical dimensions of the structure are listed in Table 1. Antenna#1 is used for transmitting (Tx), and Antenna#2 receiving (Rx). The 1×2 array is implemented on a single layer of commercially available dielectric substrate with thickness of 1.6 mm, dielectric constant of 4.3, and tanδ = 0.025. In this theoretical study the loss was not the main concern as we wanted to demonstrate the concept of meta-surface wall isolator. In practice, a low loss substrate would be employed at THz. The two square microstrip patches are excited through 50  microstrip lines as shown in Fig. 1. The meta-surface wall isolator structure was determined from extensive simulation analysis. MSWI is placed between the radiating patches designed to operate in the THz-band from 139 GHz to 141 GHz.

(a) Top and back view without MSWI					(b) Meta-surface wall isolator

(c) Isometric view with MSWI					(d) Back view with MSWI

Fig. 1. Layout of the antenna array with and without the proposed meta-surface wall isolator. Dimensions of the numerical labels in (c) and (d) are given in Table 1.

Table 1. Structural parameters of the antenna array and MSWI defined in Fig. 1

| #1 | #2 | #3 | #4 | #5 | #6 | #7 |
| --- | --- | --- | --- | --- | --- | --- |
| 5 mm | 2.5 mm | 0.625 mm | 0.25 mm | 1.0 mm | 9 mm | 4.25 mm |

S-parameter response of the proposed antenna array without and with the proposed MSWI is shown in Fig. 2. It is clear from this graph that mutual coupling is dramatically reduced after applying MSWI. With MSWI, mutual coupling is suppressed by -63.5 dB at 139 GHz, by -65 dB at 140 dB, and by -66.5 dB at 141 GHz.  Compared with no MSWI the maximum, minimum and average reduction in mutual coupling are 13.5 dB, 10 dB, and 6.5 dB, respectively. The results were obtained from 3D full-wave EM simulators of CST Microwave Studio and HFSS. Both EM tools use very different techniques to analyse the proposed structure. In fact, CST Microwave Studio uses Method of Moments discretization with a surface integral formulation of the electric and magnetic field; and HFSS uses Finite Element Method to arrive at frequency domain solution. There is excellent coherency in the results of the two very different techniques that validates the proposed technique. Salient results are summarized in Table 2.

(a) Reflection-coefficient

(b) Transmission-coefficient

Fig. 2. Reflection (S11) and transmission-coefficient (S12) response of the proposed antenna array before and after implementing MSWI.

The decoupling effects can also be observed by visualizing the surface current plots over the 1×2 antenna array, as shown in Fig. 3. The surface current density distribution in Fig. 3 show that with no MSWI and when Antenna#1 is excited the electromagnetic energy is coupled to Antenna#2, and vice versa. However, when MSWI is placed between the two antennas, it significantly chokes electromagnetic energy from Antenna#1 being coupled to Antenna#2.

Table 2. Isolation between Tx/Rx antennas.

| ¦S11¦≤-10 dB | 139 - 141 GHz (BW: 2 GHz) |  |  |
| --- | --- | --- | --- |
| S12 | Minimum | Maximum | Average |
| Without MSWI | -50 dB @ 139 GHz | -60 dB @ 141 GHz | -55 dB @ 140 GHz |
| With MSWI | -63.5 dB @ 139 GHz | -66.5 dB @ 141 GHz | -65 dB @ 140 GHz |
| Improvement | 13.5 dB | 6.5 dB | 10 dB |

Left-hand Antenna#1 is excited					          Right-hand Antenna#2 is excited

(a) Without MSWI

Left-hand Antenna#1 is excited					          Right-hand Antenna#2 is excited

(b) With MSWI

Fig. 3. Surface current density distributions over the antenna array at 140 GHz.

The antenna array’s radiation patterns for both cases of without and with MSWI structure at the mid-band frequency of 140 GHz are plotted in Fig. 4. These results show no obvious degradation in the radiation characteristics of the array radiation without and with the isolation wall. In fact, it is evident that with MSMI the radiation becomes more directive. The MSWI could be inserted retrospectively to reduce mutual coupling in planar antennas. The array antenna’s radiation gain without and with the isolation wall varies from 8.06 dBi to 9.20 dBi, and from 8.06 dBi to 8.99 dBi, respectively.

(a) Without MSWI									(a) With MSWI

Fig. 4. Radiation patterns of the antenna array in the H-plane without and with MSWI at 139 GHz, 140 GHz, and 141 GHz.

In Table 3, the proposed technique is compared with other techniques reported to date. The results, shows that the proposed mutual coupling reduction technique permits operation over a wide frequency bandwidth of 2 GHz from 139 – 141 GHz. The results show with the proposed MSWI offers higher isolation between antennas and does not degrade the antenna array’s radiation pattern. The proposed MSWI is of a simple 2D construction, which can be applied retrospectively to existing planar array antennas subject to sufficient space between the radiators. With the proposed technique the edge-to-edge gap between the transmit and receive patch antennas can be reduced to 2.5 mm. In addition, unlike other techniques the proposed technique does not require any via holes and/or defected ground-plane.

Table 3. Comparison of the proposed antenna array characteristics with recent works.

| Refs. | Method | Max. isolation improvement | Edge-to-edge separation (mm) | Freq. range (Bandwidth) | Rad. pattern deterioration | Simplicity of Design |
| --- | --- | --- | --- | --- | --- | --- |
| [14] | EBG | 4 dB | 45 mm | Narrow 5.55-5.7 GHz (150 MHz) | Yes | Low |
| [15] | UC-EBG | 10 dB | 25 mm | Narrow 5.65-5.75 GHz (100 MHz) | Yes | Low |
| [16] | U-shaped resonator | 10 dB | 25 mm | Narrow 2.38-2.5 GHz (120 MHz) | Yes | Moderate |
| [17] | SCSRR | 10 dB | 15 mm | Narrow 4.9-5.1 GHz (200 MHz) | Yes | Low |
| [18] | Meander-line resonator | 10 dB | 6 mm | Narrow 2.7-2.83 GHz (130 MHz) | No | Low |
| This work | MSWI | 13.5 dB | 2.5 mm | Wide 139-141 GHz (2 GHz) | No | High |

- CONCLUSION

A novel meta-surface wall is shown to reduce mutual coupling between adjacent microstrip patch antennas implemented on the same substrate and operating in the THz-band of 139-141 GHz. This is achieved by inserting the meta-surface wall between the radiating elements. The meta-surface wall essentially blokes the propagation of surface waves. The meta-surface wall is constructed with unit-cells comprising conjoined twin Y-shaped structure that are inter-digitally coupled to each other to create a vertical stack. With the proposed technique the isolation level of less than -63 dB is achieved between antennas. This technique can be applied to densely packed antennas in antenna arrays where the effect of mutual coupling can severely affect the array’s performance.

REFERENCES

[1] H.-S. Jang, W.-G. Lim, W.-I. Son, S.-Y. Cha, and J.-W. Yu, “Microstrip Patch Antenna with High Isolation Characteristic,” Microwave and Opt. Technol. Lett., vol. 54, no. 4, pp. 973-976, 2012.

[2] Y. Gou, S. Yang, Q. Zhu, and Z. Nie, “A Compact Dual-Polarized Double -Shaped Patch Antenna with High Isolation,” IEEE Trans. Ant. and Propag., vol. 61, no. 8, pp. 4349-4353, 2013.

[3] J. Lu, Z. Kuai, X. Zhu, and N. Zhang, “A High-Isolation Dual-Polarization Microstrip Patch Antenna with Quasi-Cross-Shaped Coupling Slot,” IEEE Trans. Ant. and Propag., vol. 59, no. 7, pp. 2713-2717, 2011.

[4] Y. Chung, S.-S. Jeon, D. Ahn, J.-I. Choi, and T. Itoh, “High Isolation Dual-Polarized Patch Antenna Using Integrated Defected Ground Structure,” IEEE Microwave and Wireless Comp. Lett., vol. 14, no. 1, pp. 4-6, 2004.

[5] S. Li, X. Cao, J. Gao, P. Gao, “High-Isolation Dual-Polarized Microstrip Antenna via Substrate Integrated Waveguide Technology,” RADIOENGINEERING, vol. 23, no. 4, pp. 1092-1098, 2014.

[6] T.-W. Chiou and K.-L. Wong, “Broad-Band Dual-Polarized Single Microstrip Patch Antenna with High Isolation and Low Cross Polarization,” IEEE Trans. Ant. and Propag., vol. 50, no. 3, pp. 399-401, 2002.

[7] X.-J. Lin, Z.-M. Xie, and P.-S. Zhang, “High Isolation Dual-Polarized Patch Antenna with Hybrid Ring Feeding,” Int. Journal of Antennas and Propagation, vol. 2017, Article ID 6193102. https://doi.org/10.1155/2017/6193102.

[8] H. Saeidi-Manesh, S. Karimkashi, G. Zhang, and R.J. Doviak, “High-Isolation Low Cross-polarization Phased Array Antenna for MPAR Application,” Radio Science, vol. 52, issue 12, pp. 1544-1557, 2017.

[9] Y.J. Park, A. Herschlein, and W. Wiesbeck, “A Photonic Bandgap (PBG) Structure for Guiding and Suppressing Surface Waves in Millimeter-Wave Antennas,” IEEE Trans. on Microwave Theory and Techniques, vol. 49, no. 10, pp. 1854–1859, 2001.

[10] D. Sievenpiper, L. Zhang, R.F.J. Broas, N.G. Alexopolous, and E. Yablonovitch, “High-Impedance Electromagnetic Surfaces with a Forbidden Frequency Band,” IEEE Trans. on Microwave Theory and Techniques, vol. 47, no. 11, pp. 2059–2074, 1999.

[11] P. S. Kildal and A. Kishk, “EM Modeling of Surfaces with Stop or Go Characteristics—Artificial Magnetic Conductors and Soft and Hard Surfaces,” Applied Computational Electromagnetics Society Journal, vol. 18, no. 1, pp. 32–40, 2003.

[12] M. Alibakhshikenari, M. Vittori, S. Colangeli, B. S. Virdee, A. Andújar, J. Anguera, and E. Limiti, “EM Isolation Enhancement Based on Metamaterial Concept in Antenna Array System to Support Full-Duplex Application,” 2017 IEEE Asia Pacific Microwave Conference (APMC2017), pp. 740-742, 13-16 Nov 2017, Kuala Lumpur, Malaysia.

[13] P. S. Kildal, A. A. Kishk, and S. Maci, “Special Issue on Artificial Magnetic Conductors, Soft/Hard Surfaces, and Other Complex Surfaces,” IEEE Trans. Ant. and Propag., vol. 53, no. 1, part 1, pp. 2–7, 2005.

[14] A. Yu, and X. Zhang, “A novel Method to Improve the Performance of Microstrip Antenna Arrays Using a Dumbbell EBG Structure,” IEEE Antennas Wireless Propagation Letters, vol. 2, no. 1, pp. 170-172, 2003.

[15] H.S. Farahani, M. Veysi, M. Kamyab, and A. Tadjalli, “Mutual Coupling Reduction in Patch Antenna Arrays Using a UC-EBG Superstate,” IEEE Antennas Wireless Propagation Letters, vol. 9, pp. 57-59, 2010.

[16] S. Farsi, D. Schreurs, and B. Nauwelaers, “Mutual Coupling Reduction of Planar Antenna by Using a Simple Microstrip U-Section,” IEEE Antennas Wireless Propag. Letters, vol. 11, pp.1501-1503, 2012.

[17] M.M.B. Suwailam, O.F. Siddiqui, and O.M. Ramahi, “Mutual Coupling Reduction Between Microstrip Patch Antennas Using Slotted-Complementary Split-Ring Resonators,” IEEE Antennas Wireless Propagation Letters, vol. 9, pp. 876-878, 2010.

[18] J. Ghosh, S. Ghosal, D. Mitra, and S.R.B. Chaudhuri, “Mutual Coupling Reduction Between Closely Placed Microstrip Patch Antenna Using Meander Line Resonator,” Progress in Electromagnetic Research Lett., vol. 59, pp.115-122, 2016.