Chapter 5

Statistical Modelling of Fatal Accidents with Fatalities and Involved Vehicles

5.0: Introduction

This chapter is based on statistical modelling of secondary data of road traffic fatal accidents with fatal casualties (i.e., fatalities) and involved vehicles following on from previous chapter 4. Statistical models will be constructed by using the two-way and three-way classified data of fatal accidents, fatal casualties (fatalities) and involved vehicles at fatal accident applying a range of statistical methods utilising a selection of predictor explanatory variables.

This chapter is organised in the following sections:

Section- 5.1

Statistical Modelling Approaches

The modelling approach to examining two-way and three-way classified data of fatal accident rates and fatality rates are to investigate the number of fatal accident, the number of fatalities and the number of involved motorised and non motorised vehicles (at fatal accidents) as the dependent variables. A Poisson regression approach is suitable for this, or when accident/ fatality rates are of a relatively small magnitude (White, 2002). However, the population density bases used at location/ attribute level and relatively long time period for which the data are available have resulted in rates that approach normally distributed data.

Population Density (PD) per square kilometres was used as population. For localities, PD by urban and locality is computed. For roads, PD by Bangladesh is computed. For divisions/ cities, PD by 10 divisions/ cities is computed. For districts/ cities, PD by 68 districts/ cities is computed. For cities-non-cities, PD by non-cities and cities (i.e., Dhaka+Chittagong+Khulna+Rajshahi) is computed. For other variables, PD by Bangladesh is computed.

The modelling flowchart is laid out below:

Fig- 5.1: Modelling Chart of Data Type

To do the modelling, SAS 9.0 is used. The procedure GENMOD is used to fit the Poisson regressions. A Poisson regression is fitted by using, in the model options as dist = poisson; link = log (this is the default link when the distribution is stated as Poisson); offset = <log of appropriate variable>. The offset is entered as the log value (e.g., log(population density) = In(population density)).

The following is a sample of the programmes:

libname RTACB "E:\RTACB Database in SAS";

data RTACB;

set RTACB.FACTDivECC;

ln_PD = log(PD);

run;

proc contents data=RTACB; run;

proc genmod data = RTACB;

class CT DivECC;

model FACTDivECC = CT DivECC Year CT*DivECC

/dist=poisson link = log offset = ln_PD type1 type3 scale = d;

*output out=RTACB1 predicted = P1;

run;

Next five sections (5.2 - 5.6) are for modelling and the last section (5.7) is for conclusion of this chapter.

Section- 5.2

Modelling of Fatalities by Travel Mode and by Age and Gender

This section is based on modelling of frequencies of traffic fatalities by travel mode, 2002-2007. The fatalities by travel mode and by age and gender are modelled. Then the fatalities for all road users, pedestrians and passengers by age and by gender for 2002-2007; drivers by age and by ‘alcohol drinking’ for 2004-2007; motorcyclists by age and by ‘helmet wearing’ for 2004-2007 are modelled. The eight models are summarised in the following table (5.2):

| # | Poisson Model | Dependent Variable | Predictor Variable Type | Significant (p<=0.05) Coefficients |  | Comment |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Variable | Value |  |
| 1 | Fatalities by Travel mode and by Age | Fatalities | Year, Travel mode and Age | Age: 26-40 41-55 Travel mode: Pedestrian Passenger Interaction: Pedes*16-25 Pedes*26-40 Pedes*41-55 | 1.5246 1.1941 2.9051 1.2351 1.9389 2.1005 1.6353 | The model is fitted well with scaled χ2/dfof 1.0577 which is only slightly overdispersed. It confirms the influences of age and travel mode. It identifies that the age of 26-40 and 41-55 are having significantly the highest fatalities over the 6-year period for all travel modes. The pedestrians are the most dangerous victims significantly. The interaction of pedestrian aged 26-40 has the highest involvement significantly (See whole analysis of parameter estimates in Appendix table- 5.2.0). |
| 2 | Fatalities by Travel mode and by Gender | Fatalities | Year, Travel mode and Gender | Gender: Male Travel mode: Pedestrian Interaction: Pedes*Male | 1.8968 0.8300 -0.5069 | The model is fitted well with scaled χ2/dfof 0.9932 which is only slightly underdispersed. It confirms the influences of gender and travel mode. It identifies that males are having significantly higher fatalities than females over 6-year period. The pedestrians are more dangerous victims than passengers significantly. The interaction of pedestrian with male has significant –ve coefficient (See whole analysis of parameter estimates in Appendix table- 5.2.0.1). |
| 3 | Fatalities by Travel mode; by Age and by Gender | Fatalities | Year, Travel mode, Age and Gender | Age: 00-15 16-25 Travel mode: Pedestrian Gender: Male Interaction: Pedes*16-25 Pedes*26-40 Pedes*41-55 Male*16-25 Male*26-40 | -0.5760 -0.4738 0.8105 0.5704 -0.7082 -0.7679 -0.5188 0.3323 0.3633 | The model is fitted well with scaled χ2/dfof 1.0306 which is only slightly overdispersed. It confirms the influences of age, travel mode and gender. Male pedestrians are the most dangerous victims significantly. The coefficients of interaction of pedestrian with ages of 26-40 are negative significantly. There is no significant coefficient in three-way classified interaction (See whole analysis of parameter estimates in Appendix table- 5.2.0.2). |
| 4 | Fatalities for All Road Users by Gender and by Age | Fatalities of All Road Users | Year, Gender and Age | Age: 00-15 16-26 Gender: Male Interaction: No | -0.6392 -0.7611 0.6004 | The model is fitted well with scaled χ2/dfof 1.0295 which is only slightly overdispersed. It confirms the influences of age and gender. It identifies that the age of 16-25 and 0-15 are having significantly the lowest fatalities over the 6-year period for all road users. The males are higher victimised than female significantly. The interaction terms are not significant (See whole analysis of parameter estimates in Appendix table- 5.2.1). |
| 5 | Fatalities for Pedestrian by Gender and by Age | Fatalities of Pedestrian | Year, Gender and Age | Age: 00-15 16-25 26-40 41-55 Gender: Male Interaction: Male*00-15 | -0.6696 -1.1798 -0.6520 -0.3724 0.6166 -0.2902 | The model is fitted well with scaled χ2/dfof 1.0549 which is only slightly overdispersed. It identifies that the age of 16-25and 0-15 have significantly the lowest fatalities over the 6-year period for pedestrians. Males are more involved than female significantly. The only interaction of male aged 0-15 is significant (See whole analysis of parameter estimates in Appendix table- 5.2.2). |
| 6 | Fatalities for Passenger by Gender and by Age | Fatalities of Passenger | Year, Gender and Age | Age: 00-15 26-40 41-55 Gender: Male Interaction: Male*16-25 Male*26-40 | -0.4899 0.2227 0.8810 0.5244 0.5117 0.5460 | The model is fitted well with scaled χ2/dfof 0.9967 which is only slightly underdispersed. It identifies that the age of 26-40 and 41-55 have significantly the highest involvement in fatalities over the 6-year period for passengers. The interaction terms of male aged 16-25 and 26-40 are significant (See whole analysis of parameter estimates in Appendix table- 5.2.3). |
| 7 | Fatalities for Driver by ‘Alcohol Drinking’ and by Age | Fatalities of Driver | Year, ‘Alcohol Drinking’ and Age | Age: 00-15 16-25 26-40 41-55 Alcohol: Suspected Interaction: Sus*00-15 Sus*16-25 Sus*26-40 | 8.8089 11.3601 12.3972 11.4700 -10.7853 9.1192 9.6096 9.7414 | The model is fitted well with scaled χ2/dfof 0.7929 which is only slightly underdispersed. It identifies that the age of 26-40and 16-25 have significantly the highest fatalities over the 4-year period for drivers. The alcohol non-suspected drivers are significantly more than suspected. The interaction terms of suspected ageing 26-40 and 16-25 are significant (See whole analysis of parameter estimates in Appendix table- 5.2.4). |
| 8 | Fatalities for Motorcyclist by ‘Helmet Wearing’ and by Age | Fatalities of Motorcyclist | Year, ‘Helmet Wearing’ and Age | Age: 16-25 26-40 41-55 Helmet: Worn Interaction: Worn*16-25 Worn*26-40 | 11.9312 13.0361 12.4873 -11.5870 10.1017 10.6140 | The model is fitted well with scaled χ2/dfof 1.1160 which is overdispersed. It identifies that the ages 26-40, 41-55 and 16-25 have significantly the highest fatalities over the 4-year period for motorcyclists. The helmet not-worn motorcyclists are significantly higher than the worn. The interactions of helmet worn ageing 26-40 and 16-25 are significant (See whole analysis of parameter estimates in Appendix table- 5.2.5). |

Table-5.2: Summary of Poisson Models of Travel Mode Fatalities by Gender and by Age

Poisson regression (full models) produced every similar result in terms of identifying age and gender factor associated with fatalities. Models were constructed to predict the fatalities of four travel modes. All of these covered the general population density of Bangladesh; all road users, pedestrians, passengers, drivers and motorcyclists. The age and gender, in which the fatalities have existence, are significant factors for certain travel mode. Findings indicate that fatalities in males are higher than in females for all travel modes. Ages are different, in that a significant difference in driver and motorcyclist fatalities was observed. Fatalities of driver from youth are significantly higher than in old. Also motorcyclist fatalities from youth are significantly higher than in old. Regression indicates that pedestrian fatalities are significantly higher in children and old person than in youth. The unknown aged fatalities were removed before modelling as this might affect in coefficients of known ages.

Individuals don’t become traffic fatalities without fatal accidents. Next three sections (5.3 - 5.5) will be for modelling of traffic fatal accidents by collision Type; junction type and mode of time.

Section- 5.3

Modelling of Fatal Accidents by Collision Type and by Location

This section is based on modelling of frequencies of fatal accidents by collision type, 2002-2007. Fatal accidents by collision type and by road environment/ locality, road class, division (excluding city)/ city, district (excluding city)/ city and city-non-city are modelled. Then, fatal accidents by collision type and by national highway link for 2003-2006 are modelled. The six models are summarised in the following table (5.3):

| # | Poisson Model | Dependent Variable | Predictor Variable Type | Significant (p<=0.05) Coefficients |  | Comment |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Variable | Value |  |
| 1 | Fatal Accident by Collision and by Locality | Fatal Accident | Year, Collision and Locality | Collision Type: Hit Pedestrian Head on Overturned V Rear end Side swipe Right angle Locality Type: Urban Interaction: HO*Urban OTV*Urban | 3.3353 2.0807 1.7594 1.6479 1.0222 -1.6805 -2.2569 -0.6152 -1.1782 | The model is fitted well with scaled χ2/dfof 0.8852 which is only slightly underdispersed. It identifies that hitting pedestrian, heading on and overturning vehicle collisions have significantly the highest fatal accidents over 6-year period. Urban locality has lower accidents than rural. The interactions, i.e., at urban, heading on and overturning vehicle fatal accidents are significant where the likelihood of being involved in a fatal accident is significantly less likely (See whole analysis of parameter estimates in Appendix table- 5.3.1). |
| 2 | Fatal Accident by Collision and by Road | Fatal Accident | Year, Collision and Road | Collision Type: Hit Pedestrian Rear end Head on Road Class: National HW Interaction: HO*National HO*Regional HO*Feeder RE*Rural SS*National SS*Regional OTV*National OTV*Regional OTV*Feeder OTV*Rural HOPV*National Hit Ped*Rural | 3.8696 2.1826 1.3373 0.7645 1.3098 1.0210 0.7570 -1.2684 0.9289 1.0391 1.7367 1.5002 1.7871 1.0561 0.9122 -0.8371 | The model is fitted well with scaled χ2/dfof 0.9981 which is only slightly underdispersed. It identifies that hitting pedestrian, rearing end and heading on collisions have significantly the highest involvement of fatal accidents over 6-year period. National highway is more dangerous than other road classes. The interactions, i.e., at national highway, the heading on, side swiping, overturning vehicle and hitting object or parked vehicle; at regional highway, the heading on, side swiping and overturning vehicle; at feeder road, the heading on and overturning vehicle; at rural roads, the rearing end and hitting pedestrian fatal accidents are significant where the likelihood of being involved in a fatal accident is significantly less likely (See whole analysis of parameter estimates in Appendix table- 5.3.2). |
| 3 | Fatal Accident by Collision and by Division/ City | Fatal Accident | Year, Collision and Division/ City | Collision Type: Hit Pedestrian Head on Side swipe Rear end Division/ City: Chittagong EC Dhaka EC Rajshahi EC Sylhet Khulna EC Barisal Interaction: No | 3.6376 2.9957 2.7081 2.6027 3.3687 3.2883 3.0774 2.2810 2.2642 2.2457 | The model is fitted well with scaled χ2/dfof 1.2391 which is overdispersed. It identifies that hitting pedestrian, heading on, side swiping and rearing end have significantly the highest fatal accidents over 6-year period. The divisions, excluding cities as Chittagong, Dhaka and Rajshahi have higher fatal accidents by collision than others. There is no significance in interactions of collision and division/ city (See whole analysis of parameter estimates in Appendix table- 5.3.3). |
| 4 | Fatal Accident by Collision and by District/ City | Fatal Accident | Year, Collision and District/ City | Collision Type: Hit Pedestrian Head on Rear end Overturned V Side swipe Right angle District/ City: Chittagong DEC Sylhet Rangamati Bandarban Naogaon Mymensingh Hobiganj Faridpur Tangail Gazipur Comilla Moulavibazar Sirajganj Jhenaidah Khagrachhari Rangpur Pabna Sunamganj Rajshahi DEC Bogra Manikganj Kishoreganj Dinajpur Feni Gopalganj Noakhali Cox's Bazar Jessore Munshiganj Narayanganj Khulna DEC Rajbari Patuakhali Bhola Natore Dhaka DEC Narsingdi Nilphamari Joypurhat Madaripur Jamalpur Chaudanga Shariatpur Kushtia Dhaka City Jhalkati Maherpur Pirojpur Narail Chittagong City Khulna City | 3.6855 2.2462 2.0449 1.8491 1.2497 -1.1206 1.7638 1.6986 1.6432 1.4393 1.2319 1.1808 1.1050 1.0991 1.0572 1.0096 0.9454 0.9371 0.9133 0.9027 0.8717 0.8548 0.7741 0.7289 0.7009 0.6545 0.6190 0.6069 0.5590 0.5454 0.5436 0.5306 0.4738 0.4640 0.4296 0.4280 0.4215 0.3984 0.3879 0.3859 0.3814 0.3304 0.3275 -0.4053 -0.4170 -0.4729 -0.5422 -0.6083 -0.7188 -0.7638 -0.8077 -0.9201 -0.9841 -1.1189 -1.2621 -1.7840 -2.7754 | The model is fitted well with scaled χ2/dfof 1.3292 which is overdispersed. It identifies that hitting pedestrian, heading on and rearing end collisions have significantly the highest involvement in fatal accident over 6-year period. 51 out of 68 districts/ cities are having higher involvement in fatal accident significantly. 37 of them have +ve coefficients and remaining 14 have –ve coefficients. (See whole analysis of parameter estimates in Appendix table- 5.3.4). |
| 5 | Fatal Accident by Collision and by City- non-City | Fatal Accident | Year, Collision and Cities/ non-Cities | Collision Type: Hit Pedestrian Head on Rear end City/ non-City: Non-Cities Interaction: OTV*non-City | 4.0473 1.6550 2.5249 5.0723 1.2475 | The model is fitted well with scaled χ2/dfof 1.0029 which is only slightly overdispersed. It identifies that the collision of hitting pedestrian, rearing end and heading on have significantly the highest involvements over 6-year period. The divisions/ districts, excluding cities have higher fatal accidents than cities. The interaction of over-turning vehicle in non-cities is significant (See whole analysis of parameter estimates in Appendix table- 5.3.5). |
| 6 | Fatal Accident by Collision and by National highway | Fatal Accident | Year, Collision and National highway link | Collision Type: No Highway link: No Interaction: No |  | The model is fitted well with scaled χ2/dfof 0.8744 which is only slightly underdispersed. It identifies that no collision, no national highway link and no interaction are significant (See whole analysis of parameter estimates in Appendix table- 5.3.5). |

Table-5.3: Summary of Poisson Models of Fatal Accidents by Collision and by Location

The predominant fatal collision accident on any road (mainly national highways) and in either an urban or rural locality is a vehicle hitting pedestrian. Following fatal hitting pedestrian accidents, heading on, rearing end and overturning are the other main collisions which account for a high proportion of fatal collision accidents in the rural localities. Again, half of all fatal accidents being collisions where a vehicle hits pedestrian(s) are evident in all divisions/ Districts and cities.

Fatal accidents by junction type and by location are modelled at next section (5.4).

Section- 5.4

Modelling of Fatal Accidents by Junction Type and by Location

This section is based on modelling of frequencies of fatal accidents by junction type, 2002-2007. The fatal accidents by junction type and by road environment/ locality, road class, division (excluding city)/ city, district (excluding city)/ city and cities-non-cities are modelled. The five models are summarised in the following table (5.4):

| # | Poisson Model | Dependent Variable | Predictor Variable Type | Significant (p<=0.05) Coefficients |  | Comment |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Variable | Value |  |
| 01 | Fatal Accident by Junction and by Locality | Fatal Accident | Year, Junction and Locality | Junction Type: Not at Junction Cross Tee Off-set Tee Roundabout Railway Cross Locality Type: Urban Interaction: Cross*Urban Rounda*Urban | 2.2109 -1.2811 -0.5899 -2.2390 -3.7896 -4.8882 -2.1588 1.0113 2.2437 | The model is fitted well with scaled χ2/dfof 1.0520 which is only slightly overdispersed. All junctions having the significant involvements are more than railway crossing, except not at junction which is the most dangerous. Urban locality is less dangerous than rural. The only significant interaction as cross junction at urban makes accidents more likely. The interaction as roundabout at urban is slightly significant but having big +ve coefficient (See whole analysis of parameter estimates in Appendix table- 5.4.1). |
| 02 | Fatal Accident by Junction and by Road | Fatal Accident | Year, Junction and Road | Junction Type: Not at Junction Tee Off-set Tee Railway Cross Road Class: National HW Interaction: Not at Junc*Rural Cross*National Cross*Regiona Cross*Feeder Cross*Rural Tee*National Tee*Regional Tee*Feeder Tee*Rural Round*National | 2.3775 0.6645 -1.7762 -2.6741 1.2893 -0.9377 -1.1674 -1.6770 -1.3808 -2.3121 -1.1314 -1.1628 -1.2760 -1.4206 -2.6630 | The model is fitted well with scaled χ2/dfof 1.0384 which is only slightly overdispersed. It identifies that the junctions as not at junction, tee and Cross; the road class as national highways have significantly the highest involvement over 6-year period. The interaction as not at junction in rural road has the highest involvement (See whole analysis of parameter estimates in Appendix table- 5.4.2). |
| 03 | Fatal Accident by Junction and by Division/ City | Fatal Accident | Year, Junction and Division/ City | Junction Type: Not at Junction Division/ City: Chittagong EC Sylhet Dhaka EC Khulna EC Rajshahi EC Chittagong City Khulna City Interaction: No | 2.1712 3.9433 2.9619 2.0996 2.1328 3.2259 -2.0296 -2.2490 | The model is fitted with scaled χ2/dfof 1.4154 which is overdispersed. It indicates that not at junction has only significant involvement and this is the highest. Chittagong division (excluding city) has the highest and Khulna city has the lowest involvement based in Rajshahi City. Cities have lower fatalities than divisions, excluding cities. The significant interactions are absent (See whole analysis of parameter estimates in Appendix table- 5.4.3). |
| 04 | Fatal Accident by Junction and by District/ City | Fatal Accident | Year, Junction and District/ City | Junction Type: Not at Junction Cross Tee Off-set Tee Roundabout Railway Cross District/ City: Chittagong DEC Sylhet Rangamati Bandarban Naogaon Mymensingh Hobiganj Faridpur Tangail Gazipur Comilla Moulavibazar Sirajganj Jhenaidah Khagrachhari Rangpur Pabna Sunamganj Rajshahi DEC Bogra Manikganj Kishoreganj Dinajpur Feni Gopalganj Noakhali Cox's Bazar Jessore Munshiganj Narayanganj Jamalpur Chaudanga Shariatpur Kushtia Dhaka City Jhalkati Maherpur Pirojpur Narail Chittagong City Khulna City | 2.1881 -0.7584 -0.2668 -2.0263 -2.6844 -4.0593 1.7656 1.6986 1.6432 1.4393 1.2319 1.1872 1.1050 1.0991 1.0825 1.0096 0.9398 0.9371 0.9133 0.9027 0.8717 0.8548 0.7741 0.7289 0.7009 0.6545 0.6190 0.6069 0.5590 0.5454 0.5436 0.5306 0.4738 0.4640 0.4296 0.4280 -0.5422 -0.6083 -0.7188 -0.7638 -0.8195 -0.9201 -0.9841 -1.1189 -1.2621 -1.7862 -2.7754 | This model is fitted with scaled χ2/dfof 1.1810 which is over dispersed. Not at junction and railway crossing are having the highest and the lowest involvements respectively. 41 out 68 districts (excluding cities)/ cities are significant based in Thakurgaon. 30 of them have +ve coefficients and remaining 11 have –ve coefficients. The cities of Dhaka, Chittagong and Khulna have lower involvement significantly than districts (excluding cities). (See whole analysis of parameter estimates in Appendix table- 5.4.4). |
| 05 | Fatal Accident by Junction and by Cities/ non-cities | Fatal Accident | Year, Junction and City/ non-city | Junction Type: Not at Junction Cross Tee City-non-city: Non-cities Interaction: Cross*Non-Cities Tee*Non-Cities Rounda*Non-Cities | 2.8518 0.7677 1.1635 5.3289 -1.7504 -1.1920 -3.1978 | The model is fitted well with scaled χ2/dfof 1.0577 which is only slightly overdispersed. The not at junction and tee junction are having the highest involvement in fatal accidents. Non-cities are more dangerous than cities. The interaction of cross junction in non-cities has maximum involvement significantly (See whole analysis of parameter estimates in Appendix table- 5.4.5). |

Table-5.4: Summary of Poisson Models of Fatal Accidents by Junction Type and by Location

The predominant fatal junction accident on any road and in either an urban or rural locality is a vehicle not at junction accident. Following fatal not at junction accidents, tee, cross and off-set tee are the other main junctions which account for a high proportion of fatal junction accidents in the rural localities. In all divisions and cities, 2/3 of all fatal accidents are not at junction. Most of these fatal junction accidents occur in Chittagong Division, Dhaka division and Rajshahi division, outside of city areas.

Fatal accidents by time mode and by location are modelled at next section (5.5).

Section- 5.5

Modelling of Fatal Accidents by Time and by Road Environment and Class

This section is based on modelling of frequencies of fatal accidents by time mode, 2004-2007. Fatal accidents by time, day and month and each by locality and road are modelled. The results of six models are summarised in the following table (5.5):

| # | Poisson Model | Dependent Variable | Predictor Variable Type | Significant (p<=0.05) Coefficients |  | Comment |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Variable | Value |  |
| 01 | Fatal Accident by Time and by Locality | Fatal Accident | Year, Time and Locality | Time: 01-02 05-06 07-08 09-10 11-12 13-14 15-16 17-18 19-20 21-22 Locality Type: Urban Interaction: Urban*01-02 Urban*07-08 Urban*09-10 Urban*11-12 Urban*13-14 Urban*15-16 Urban*17-18 Urban*19-20 | 0.6528 0.4771 0.9166 1.0851 1.3214 1.0871 0.8955 0.9902 0.6080 0.4580 -0.7282 -0.5117 -0.3458 -0.4610 -0.5139 -0.5370 -0.6124 -0.5510 -0.3764 | The model is fitted well with scaled χ2/dfof 1.0166 which is only slightly overdispersed. It identifies that the time of 09-10, 11-12 and 13-14 have significantly the highest fatal accidents over 4-year period. All time variables have significant coefficients except 03-04. Urban locality has lower involvement than rural. Interaction terms as urban on 07-08 and 09-10 have higher involvement based on 23-24. (See whole analysis of parameter estimates in Appendix table- 5.5.1.1) |
| 02 | Fatal Accident by Time and by Road | Fatal Accident | Year, Time and Road | Time: 01-02 05-06 07-08 09-10 11-12 13-14 15-16 17-18 19-20 21-22 Road Class: National HW Regional HW Rural road Interaction: 03-04*Rural | 0.6637 0.3973 0.7668 1.0083 1.1718 0.9417 0.8566 0.9281 0.4882 0.3466 1.1499 -0.6328 -0.6884 -0.0872 | The model is fitted well with scaled χ2/dfof 0.9954 which is only slightly underdispersed. It identifies that the time 11-12 and 09-10; the national highways have significantly the highest fatal accidents over 4-year period. The only significant interaction is rural road on 03-04 based on 23-24 (See whole analysis of parameter estimates in Appendix table- 5.5.1.2). |
| 03 | Fatal Accident by Day and by Locality |  | Year, Day and Locality | Day: No Locality Type: Urban Interaction: No | -1.1701 | The model is fitted well with scaled χ2/dfof 1.0113 which is only slightly overdispersed. It identifies that no day is significant over 4-year period. Urban locality has lower significant involvement than rural. No interaction is significant based on Sunday (See whole analysis of parameter estimates in Appendix table- 5.5.2.1). |
| 04 | Fatal Accident by Day and by Road | Fatal Accident | Year, Day and Road | Day: No Road Class: National HW Rural road Interaction: No | 1.0054 -0.7504 | This model is fitted well with scaled χ2/dfof 0.9528 which is only slightly underdispersed. It identifies that no day is significant over 4-year period. National highway has higher significant involvement than other. No interaction is significant based on Sunday (See whole analysis of parameter estimates in Appendix table- 5.5.2.2) |
| 05 | Fatal Accident by Month and by Locality | Fatal Accident | Year, Month and Locality | Month: No Locality: Urban Interaction: No | -1.1888 | This model is fitted well with scaled χ2/dfof 0.9862 which is only slightly underdispersed. It identifies that urban locality is less dangerous over 4-year period. No month is significant and also no interaction is significant based on December (See whole analysis of parameter estimates in Appendix table- 5.5.3.1). |
| 06 | Fatal Accident by Month and by Road | Fatal Accident | Year, Month and Road | Month: September Road Class: National HW Regional HW Rural road Interaction: No | -0.1937 0.9734 -0.2781 -0.4622 | This model is fitted well with scaled χ2/dfof 0.9624 which is only slightly underdispersed. It identifies that the month of September; the roads of national highways have significantly the highest involvement in fatal accidents over 4-year period. Here is no significant interaction based on December (See whole analysis of parameter estimates in Appendix table- 5.5.3.2). |

Table-5.5: Summary of Poisson Models of Fatal Accidents by Time and by Location

Maximum fatal accidents occur at day time with highest at 08:01-14:00 in either rural or urban localities (rural > urban) or any roads (highest in national highways). Sundays, Fridays and Tuesdays have the highest fatal accidents in either rural or urban localities or any roads. Again, January, March, May, April and December are having the highest fatal accidents either in rural or urban localities or any roads.

There are no traffic fatalities or fatal accidents without vehicles. Involved vehicles at fatal accidents are modelled at Next section (5.6).

Section- 5.6

Modelling of Involved Vehicles by Road and by Collision Type

This section is based on modelling of frequencies of involved vehicles at fatal accidents, 2002-2007. The involved vehicles by type; by vehicle per fatal accident and by road environment/ locality, road class and collision type are modelled. Then, these vehicles by type and by road environment, road class and collision type for single (1) vehicle and multiple (1+) vehicles per accident are modelled. The results of ten models are summarised in the following table (5.6):

| # | Poisson Model | Dependent Variable | Predictor Variable Type | Significant (p<=0.05) Coefficients |  | Comment |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Variable | Value |  |
| 01 | Involved Vehicle by Type and by Vehicle per accident | Involved Vehicle | Year, Vehicle- type and Vehicle per Accident | Vehicle Type: Bicycle Rickshaw Motorcycle Baby-Taxi Tempo Microbus Minibus Bus Heavy Truck Articulated Truck Oil Tanker Vehicle per Accident: Single Interaction: Bicycle*Single Rickshaw*Single Motorcycle*Single BabyTaxi*Single Tempo*Single | 1.1572 1.4315 1.3106 0.5789 0.8033 0.4078 1.2767 2.3017 2.3857 -1.4220 -1.5997 0.5883 -3.2651 -5.4112 -1.1210 -1.1913 -0.7487 | The model is fitted well with scaled χ2/dfof 0.9552 which is only slightly underdispersed. It identifies that vehicles of heavy truck, bus, Rickshaw, Motorcycle and minibus have significantly the highest involvement over 6-year period. Single vehicle per fatal accident has higher involvement than multiple vehicles per fatal accident. The interaction term of tempo in single vehicle per fatal accident is higher significant than other significant terms based on other vehicles (See the whole analysis of parameter estimates in Appendix table- 5.6.1.1). |
| 02 | Involved Vehicle by Type; by Vehicle per accident and by Locality | Involved Vehicle | Year, Vehicle- Type; Vehicle per accident and Locality | Vehicle Type: Bicycle Rickshaw Motorcycle Baby-Taxi Tempo Minibus Bus Truck Heavy Truck Articulated Truck Oil Tanker Vehicle per Accident: Single Locality: Urban Interaction: Bicycle*Single Rickshaw*Single Motorcycle*Single Baby Taxi*Single Tempo*Single Bus*Single | 1.1043 1.1436 1.3118 0.5706 0.8805 1.2509 2.3581 0.5403 2.3884 -1.5174 -1.4507 0.5021 -1.9721 -2.9357 -4.9209 -0.9271 -1.3623 -0.6078 -0.7603 | The model is fitted well with scaled χ2/dfof 0.9559 which is only slightly underdispersed. It identifies that vehicles of heavy truck, bus, Rickshaw, Motorcycle and minibus have significantly the highest involvement over 6-year period. Single vehicle per fatal accident has higher involvement than multiple vehicles per fatal accident Rural locality has larger involvement of vehicles than that in urban. The interaction term of tempo in single vehicle per fatal accident is higher significant than other significant terms. No two-way classified interactions of vehicle type by locality and vehicle per fatal accident by locality; three-way classified interactions of vehicle type by vehicle per FA and by locality are significant (See the whole analysis of parameter estimates in Appendix table- 5.6.1.2). |
| 03 | Involved Vehicle by Type; by Vehicle per accident and by Road | Involved Vehicle | Year, Vehicle- Type; Vehicle per accident and Road | Vehicle Type: Bicycle Rickshaw Motorcycle Minibus Bus Heavy Truck Vehicle per Accident: Single Road Class: National highway Rural road Interaction: Regional HW*Single Bicycle*Single Motorcycle*Single Baby Taxi*Single National *Rickshaw National *Bus National *Hev Truck Regional *Rickshaw Regional *Car Feeder *Tempo Feeder *Car Rural * Motorcycle Rural *Tempo Rural *Tractor | 0.8786 2.0607 0.7115 1.2993 1.8341 2.0320 0.9946 1.0831 -1.0445 -0.7822 -3.7827 -1.5054 -1.0144 -0.7732 0.8867 0.6547 -0.9345 -1.4168 1.2654 -1.4469 1.1316 1.4092 3.3471 | The model is fitted well with scaled χ2/dfof 1.0399 which is only slightly underdispersed. It identifies that vehicles of heavy truck, bus, Rickshaw and minibus have significantly the highest involvement over 6-year period. Single vehicle per fatal accident has higher involvement than multiple vehicles per fatal accident. National highways have larger involvement of vehicles than any roads. The interaction term of tempo in single vehicle per fatal accident is higher significant than other significant terms. No three-way classified interactions of vehicle type by vehicle per FA and by road are significant (See the whole analysis of parameter estimates in Appendix table- 5.6.1.3). |
| 04 | Involved Vehicle by Type; by Vehicle per accident and by Collision | Involved Vehicle | Year, Vehicle- Type; Vehicle per accident and Collision | Vehicle Type: Bicycle Bus Heavy Truck Vehicle per Accident: Single Collision Type: Head on Rear end Side swipe Interaction: Head on*Single Rear end*Single Side swipe*Single HOPV*Single Hit pedestrian*Single Bicycle*Single Rickshaw*Single Oil tanker*Single Head on*Motorcycle Head on*Tempo Head on*Minibus Head on*Bus Head on*Hev truck Rear end*Rickshaw Side swipe*Minibus Oil tanker*HO*Single Bicycle*RE*Single Oil tanker*RE*Single Bicycle*SS*Single Car*SS*Single Oil tanker*SS*Single Bicycle*OTV*Single Oil tanker*OTV*Single Bicycle*HOPV*Single Car*HOPV*Single | 1.1527 1.6422 1.7918 1.2528 2.7983 2.9223 1.8056 -5.8428 -4.7140 -4.1569 -2.4399 2.4196 -21.4054 -3.8918 -18.4609 1.3159 2.2256 2.2056 1.2201 0.9922 1.2276 2.0552 20.5661 19.2831 20.5359 20.7353 3.7106 19.8610 17.3363 17.3876 19.8200 3.1723 | The model is fitted well with scaled χ2/dfof 0.9552 which is only slightly underdispersed. It identifies that vehicles of heavy truck, bus, and bicycle have significantly the highest involvement in fatal accidents over 6-year period. Single vehicle per fatal accident has higher involvement than multiple vehicles per fatal accident. Heading on and rearing end have larger involvement than any collision. The interaction term of rickshaw in single vehicle per fatal accident is higher significantly than other; hitting pedestrian in single vehicle per fatal accident is the highest significantly; heading on is higher for tempo than other significantly (See the whole analysis of parameter estimates in Appendix table- 5.6.1.4). |
| 05 | Involved Vehicle by Type and by Locality in Single Vehicle per accident | Involved Vehicle | Year, Vehicle- type and Locality | Vehicle Type: Bicycle Rickshaw Baby-Taxi Microbus Minibus Bus Heavy Truck Articulated Truck Oil Tanker Locality: Urban Interaction: Motorcycle*Urban Tempo*Urban Bus*Urban Truck*Urban | -1.8314 -3.7773 -0.7917 0.4654 1.2622 2.4535 2.0211 -2.5536 -1.8.33 -1.7363 -1.2578 -1.4129 -0.7045 -1.2745 | The model is fitted well with scaled χ2/dfof 0.9446 which is only slightly underdispersed. It identifies that the vehicle of bus, heavy truck, minibus and microbus have significantly the highest involvement in fatal accidents over 6-year period. Urban locality has lower involvement of vehicles than rural. The interaction of bus in urban locality is higher significant than other significant terms based on other vehicles (See whole analysis of parameter estimates in Appendix table- 5.6.2.1). |
| 06 | Involved Vehicle by Type and by Road in Single Vehicle per accident | Involved Vehicle | Year, Vehicle- type and Road | Vehicle Type: Bicycle Minibus Bus Jeep Truck Heavy Truck Articulated Truck Oil Tanker Tractor Road Class: National HW Regional HW Feeder Rural Interaction: Motorcycle*Regio Motorcycle *Feed Motorcycle *Rural Tempo*Feeder Tempo*Rural Bus*National Jeep*Rural Truck*National Truck*Regional Truck*Feeder Truck*Rural Tractor*Feeder Tractor*Rural | -2.9042 1.4589 1.9361 -1.2947 -1.2947 1.9161 -1.0716 -1.5824 -2.9042 0.6111 -0.5892 -0.5588 -0.8484 1.3967 1.8329 1.7827 1.8466 1.6312 0.6971 2.0722 1.4301 1.4646 2.1377 1.9132 3.2500 3.7662 | The model is fitted well with scaled χ2/dfof 1.0656 which is only slightly overdispersed. It identifies that the vehicle of bus, heavy truck, minibus and car have significantly the highest involvement in fatal accident over 6-year period. The national highways are more dangerous significantly than any other roads. The interaction terms of tractor in feeder and rural roads, of truck in feeder roads and jeep in rural roads have higher significances based on other vehicles (See the whole analysis of parameter estimates in Appendix table- 5.6.2.2). |
| 07 | Involved Vehicle by Type and by Collision in Single Vehicle per accident | Involved Vehicle | Year, Vehicle- type and Collision | Vehicle Type: Bicycle Minibus Bus Heavy Truck Oil Tanker Collision Type: Head on Rear end Side swipe Overturned Vehicle Hit pedestrian Interaction: Rear end*Bicycle Side swipe*Bicycle OverturnV*Bicycle HOPV*Bicycle RE*Motorcycle OTV*Tempo HOPV*Tempo RE*Microbus SS*Microbus HOPV*Microbus SS*Bus HOPV*Bus HOPV*Car HOPV*Pick-up OTV*Truck HO*Heavy Truck RE*HeavyTruck OTV*HeavTruck HOPV*Heavy Truck HO*Oil Tanker RE*Oil Tanker SS*Oil Tanker OTV*Oil Tanker HOPV*Oil Tanker Hit pedestria*Tractor | -21.2523 1.2993 2.2637 1.2595 -21.2523 -3.0445 -1.7918 -2.3514 1.0245 3.3020 20.6927 21.2523 17.8764 19.7482 2.4849 1.3321 1.6458 2.3179 3.1186 2.0665 1.9847 1.1074 2.4960 2.2336 1.1865 2.7658 2.2062 1.1422 1.8213 21.9454 21.7913 21.9454 19.4858 20.4413 -1.0878 | The model is fitted well with scaled χ2/dfof 1.1458 which is overdispersed. It identifies that the vehicle of bus, minibus, heavy truck and tractor have significantly the highest involvement in fatal accidents over 6-year period. The collision of hitting pedestrian and overturning vehicle are having higher significant involvement than any collision. The interaction of tractor by hitting pedestrian has the lowest significant involvement. The side swiping and hitting object or parked vehicle of buses is significant based on other vehicles (See the whole analysis of parameter estimates in Appendix table- 5.6.2.3). |
| 08 | Involved Vehicle by Type and by Locality in Multiple Vehicles per accident | Involved Vehicle | Year, Vehicle- type and Locality | Vehicle Type: Bicycle Rickshaw Motorcycle Baby-Taxi Tempo Microbus Minibus Bus Jeep Truck Heavy Truck Articulated Truck Oil Tanker Locality: Urban Interaction: Tempo*Urban Microbus*Urban Bus*Urban Truck*Urban Heavy Truck*Urban | 1.1043 1.1436 1.3118 0.5706 0.8805 0.4906 1.2509 2.3581 -0.7576 0.5403 2.3884 -1.5174 -1.4507 -1.9719 -0.8436 -0.9778 -0.7603 -1.0275 -0.5467 | The model is fitted well with scaled χ2/dfof 0.9680 which is only slightly underdispersed. It identifies that the vehicle of heavy truck, bus, motorcycle, minibus and rickshaw have significantly the highest involvement in fatal accidents over 6-year period. The urban locality is less dangerous than rural. The all significant interactions are negative. No interaction with non-motorised vehicle is significant based on other vehicles (See the whole analysis of parameter estimates in Appendix table- 5.6.3.1). |
| 09 | Involved Vehicle by Type and by Road in Multiple Vehicles per accident | Involved Vehicle | Year, Vehicle- type and Road | Vehicle Type: Bicycle Rickshaw Motorcycle Baby-Taxi Minibus Bus Car Heavy Truck Oil Tanker Tractor Road Class: National HW Rural Interaction: Rickshaw*National HW Rickshaw*Regional HW Rickshaw*Feed Tempo*Feeder Tempo*Rural Microbus*National Bus*National Car*Regional Car*Feeder Heavy Truck*National Tractor*Rural | 0.8786 2.0607 0.7115 0.6360 1.2993 1.8341 0.6360 2.0320 -2.6027 -2.6027 1.0831 -1.0445 -0.7732 -0.9345 -0.7181 1.2654 1.4092 1.0687 0.8267 -1.4168 -1.4469 0.6547 3.3471 | The model is fitted well with scaled χ2/dfof 0.9787 which is only slightly underdispersed. It identifies that the vehicle of rickshaw, heavy truck, bus, minibus and bicycle have significantly the highest involvement in fatal accidents over 6-year period. The national highways and rural roads are significant. The interactions of tractor in rural road; tempo in rural road; microbus, bus and heavy truck in national highways are higher significant than any based on other vehicles based on other vehicles (See the whole analysis of parameter estimates in Appendix table- 5.6.3.2). |
| 10 | Involved Vehicle by Type and by Collision in Multiple Vehicles per accident | Involved Vehicle | Year, Vehicle- type and Collision | Vehicle Type: Bicycle Bus Heavy Truck Collision Type: Head on Rear end Side swipe Hit pedestrian Interaction: Motorcycle*HO Motorcycle*SS Baby-Taxi*HO Tempo*HO Minibus*HO Minibus*SS Bus*HO Heavy Truck*HO Heavy Truck*SS | 1.1527 1.6422 1.7918 2.7983 2.9223 1.8056 0.8824 1.3159 1.2294 3.1882 2.2256 2.2056 2.0552 1.2201 0.9922 0.9245 | The model is fitted well with scaled χ2/dfof 1.1923 which is overdispersed. It identifies that the vehicle of heavy truck, bus, bicycle, rickshaw and motorcycle have significantly the highest involvement in fatal accidents over 6-year period. The collision of rearing end and heading on are highly significant. The interactions of heading on and side swiping with motorcycle, baby-taxi, tempo, minibus, bus and heavy truck are only significant based on other vehicles (See whole analysis of parameter estimates in Appendix table- 5.6.3.3). |

Table-5.6: Summary of Poisson Models of Involved Vehicles by Type and by Road Environment and Class

The predominant involved vehicles (in fatal accident) as heavy trucks, buses and minibuses are over-represented; jeep, baby-taxi and tempo are to a lesser degree. The involvement of buses and heavy vehicles (heavy truck, truck, artic truck and oil tanker) account larger in fatal single vehicle accidents than that in fatal multiple vehicles accidents. Hitting pedestrian, overturning vehicle and rear ending are main collisions in fatal single vehicle accidents on any road and in either an urban or rural locality. Heading on and rearing end are main collisions in fatal multiple vehicle accidents on any road and in either an urban or rural locality.

This chapter is concluded at next section (5.7).

Section- 5.7

Conclusion

Traffic fatal accidents, fatalities and involved vehicles have been modelled in this chapter. The main key findings are as follows:

Traffic fatalities are modelled based on travel mode (Pedestrian, Passenger, Driver, Motorcyclist and All Road User) by age and by gender or alcohol-drinking or helmet-wearing. Persons who are killed as the result of a traffic accident are predominantly young pedestrians, adult pedestrians and passengers.

Fatal accidents are modelled based on collision type and junction type, each by locality, road, division/ city and district/ city. Fatal accidents mainly occur on national highways. Predominantly, fatal accidents involve vehicles hitting pedestrians. The highest number of fatalities per fatal accident occurs when vehicle collide head on. Again, fatal accidents are modelled based on time mode (time, day and month) by locality and road. Maximum fatal accidents occur at day time in either rural or urban localities or any roads. Sundays, Fridays and Tuesdays have the highest fatal accidents in either rural or urban localities or any roads. Again, January, March, May, April and December are having the highest fatal accidents either in rural or urban localities or any roads.

Involved vehicles at fatal accidents are modelled based on type by road environment/ locality, road class and collision type in single/ multiple vehicle(s) per accident. The greatest numbers of fatal accidents by far involve heavy trucks and buses. Fatal accidents are more likely to involve multiple fatalities in sites with a high proportion of ‘cross-centreline’ or off-road type accidents resulting from dangerous overtaking and/ or over-speeding.

Therefore, policies aimed at reducing traffic accidents and casualties should be applied with caution. In terms of traffic accidents and casualties, remedial measures should be targeted at individual and location levels.

The Research is concluded with discussions and recommendations at next chapter (6).